## 1. Threat Model

| Attribute | Specification |
| --- | --- |
| Target System | milliAmpere multi-sensor fusion pipeline (YOLO v4 + JIPDA tracker) |
| Attack Goals | (a) Evade detection (miss targets), (b) Cause false tracks, (c) Degrade tracking accuracy, (d) Poison training data |
| Knowledge | White-box (full model access) → Gray-box (sensor characteristics known) → Black-box (query-only) |
| Perturbation | Physical-world feasible (camera: patch/sticker; radar/lidar: spoofing/object placement) |
| Constraints | Perturbations must be physically realizable in maritime environment |

## 2. Attack Surface Mapping

┌─────────────────────────────────────────────────────────────┐

│                    ATTACK SURFACE                           │

├─────────────────────────────────────────────────────────────┤

│  INPUT LAYER          │  PROCESSING LAYER    │  FUSION LAYER │

├─────────────────────────────────────────────────────────────┤

│  • EO/IR Images       │  • YOLO v4 Detector  │  • JIPDA      │

│  • Radar Point Clouds │  • Clustering (DBSCAN│    Association│

│  • Lidar Point Clouds │    /Hierarchical)    │  • Track Mgmt │

│  • Navigation Data    │  • Land Mask Filter   │  • Existence  │

│                       │                       │    Prob Update│

└─────────────────────────────────────────────────────────────┘

## 3. Model Evasion Attacks

### 3.1 Camera Branch (YOLO v4 — EO & IR)

| Attack Vector | Method | Implementation |
| --- | --- | --- |
| Digital Adversarial Patches | Expectation Over Transformations (EOT) + PGD | Optimize patch on boat hull/waterline that minimizes objectness score across rotations, scales, lighting |
| Physical Adversarial Patches | LaVAN/RP2-style patch | Printable sticker on target vessel hull; optimized for maritime lighting (specular reflections, wave motion) |
| Universal Perturbations | UAP for YOLO | Single noise pattern that suppresses "boat" class across multiple maritime scenes |
| Backdoor Trigger | Clean-label poisoning | Invisible IR-reflective pattern (only visible to IR camera) that causes misclassification |

Targeted Effects:

- Disappearance: Reduce objectness score below NMS threshold → no bounding box

- Misclassification: "Boat" → "Buoy" or background class

- Localization Error: Shift bounding box bottom-center → incorrect bearing measurement

Physical Constraints for Maritime:

- Patch must survive water spray, salt corrosion, wave motion

- IR attacks: exploit thermal properties (heating/cooling hull sections)

### 3.2 Radar Branch (Active Sensor)

| Attack Vector | Method | Implementation |
| --- | --- | --- |
| Ghost Target Injection | Radar spoofing (signal replay/delay) | Inject false point cloud returns at specific ranges/bearings to create fake clusters |
| Target Suppression | Coordinated noise jamming | Raise noise floor to push true returns below detection threshold |
| Cluster Poisoning | Point cloud adversarial perturbation | Add adversarially placed points around true target to break clustering (split one target into multiple sub-threshold clusters, or merge with nearby clutter) |

Mathematical Formulation:
Given radar point cloud P = {p₁, ..., pₙ}, find perturbation δ such that:

- Clustering algorithm fails: centroid(P + δ) ≠ true target position

- Or cluster size < minimum threshold (5 points) → filtered out

### 3.3 Lidar Branch (Active Sensor)

| Attack Vector | Method | Implementation |
| --- | --- | --- |
| Adversarial 3D Object | Point cloud adversarial examples (C&W / PGD for point clouds) | Modify target geometry representation to cause clustering failure |
| Spoofing | Lidar spoofing (laser injection) | Inject false points to create ghost clusters or obscure true target |
| Occlusion Amplification | Physical object placement | Small reflective object placed to exploit clustering distance threshold T |

Key Insight from Paper:
The clustering uses Euclidean distance threshold T and minimum 5-point threshold. An attacker can:

- Split attack: Insert sparse points around target to prevent density-based grouping

- Merge attack: Add points between two targets to force single merged cluster

### 3.4 Multi-Sensor Fusion Evasion (JIPDA Tracker)

This is the most sophisticated attack surface — exploit the sensor fusion logic itself.

| Attack Strategy | Mechanism | Effect |
| --- | --- | --- |
| Sensor-Specific Blind Spot Exploitation | Attack only EO camera during low-light; only IR during bright sun; only radar during heavy rain | Force tracker to rely on degraded sensor |
| Existence Probability Suppression | Prevent all sensors from detecting target for consecutive scans | Drive εᵗₖ (existence probability) below termination threshold Tᵣ |
| False Track Injection | Inject consistent ghost measurements across 2+ sensors | Create confirmed false tracks that consume computational resources and confuse downstream planning |
| Association Confusion | Cause measurements to fall outside validation gates | Force JIPDA to evaluate aᵗₖ = 0 (no measurement) hypotheses, reducing update quality |

Cross-Sensor Consistency Attack:
The JIPDA assumes independent sensor errors. An attacker who can coordinate across modalities (e.g., physical patch on target + radar spoofing) can create correlated failures that break the independence assumption, causing the fusion to perform worse than any single sensor.

## 4. Data Poisoning Attacks

### 4.1 Training-Time Poisoning (YOLO v4)

| Poisoning Type | Method | Target |
| --- | --- | --- |
| Clean-Label Poisoning | Add imperceptible perturbations to training images; keep labels correct | Cause model to learn brittle features that fail on adversarial examples at inference |
| Backdoor Poisoning | Insert trigger pattern (e.g., specific wave pattern in water) into training images with "no-object" labels | Trigger causes detector to ignore any vessel with that pattern |
| Availability Poisoning | Add label noise (swap boat/buoy labels), bounding box jitter | Degrade overall detector mAP |

Maritime-Specific Backdoor Triggers:

- Specific wake pattern behind vessel

- IR-reflective tape pattern (invisible in EO, visible in IR)

- Structured light reflection on water surface

### 4.2 Poisoning of Auxiliary Data

| Target | Method | Effect |
| --- | --- | --- |
| Land Map | Corrupt GeoJSON masks or Norwegian Mapping Authority data | Cause land filtering to fail → false tracks from shore returns |
| Calibration Parameters | Perturb camera intrinsic/extrinsic matrices | Bearing extraction error propagates to tracker |
| Navigation Data | GPS spoofing of ownship position | All NED transforms incorrect → systematic bias in fused tracks |

## 5. Attack Pipeline Architecture

┌─────────────────────────────────────────────────────────────────────┐

│                    ADVERSARIAL AI PIPELINE                          │

├─────────────────────────────────────────────────────────────────────┤

│  PHASE 1: RECONNAISSANCE                                            │

│  • Clone dataset from GitHub (scenarios 2, 3, 4, 5, 6)              │

│  • Train surrogate YOLO v4 on COCO + maritime fine-tuning           │

│  • Implement radar/lidar clustering pipeline per paper              │

│  • Reproduce JIPDA tracker (use Stone Soup / MATLAB)                │

├─────────────────────────────────────────────────────────────────────┤

│  PHASE 2: DIGITAL ATTACK DEVELOPMENT                                │

│  ├─ Camera: FGSM, PGD, C&W, EOT on EO/IR images                     │

│  ├─ Radar: Point cloud PGD on clustered centroids                   │

│  ├─ Lidar: PointNet++ adversarial points (if 3D model used)         │

│  └─ Fusion: Gradient-based attacks through JIPDA update equations   │

├─────────────────────────────────────────────────────────────────────┤

│  PHASE 3: PHYSICAL REALIZABILITY                                    │

│  ├─ EOT with maritime transforms (wave motion, specular, haze)      │

│  ├─ Printability constraints for adversarial patches                │

│  ├─ Radar/lidar: simulate with CARLA/ custom ray-tracing            │

│  └─ Validate in simulation before field test                        │

├─────────────────────────────────────────────────────────────────────┤

│  PHASE 4: EVALUATION                                                │

│  • Detection: mAP, precision/recall, RMSE vs. ground truth          │

│  • Tracking: MOTA, MOTP, ID switches, track fragmentation           │

│  • Compare: Single-sensor vs. fused under attack                    │

│  • Measure: Attack success rate vs. perturbation budget             │

├─────────────────────────────────────────────────────────────────────┤

│  PHASE 5: DEFENSE DEVELOPMENT                                       │

│  • Adversarial training on generated examples                       │

│  • Input sanitization (point cloud outlier removal)                 │

│  • Sensor consistency checks (cross-modal validation)               │

│  • Robust clustering (RANSAC-based instead of single-link)          │

## 6. Evaluation Metrics

| Layer | Benign Metric | Adversarial Metric |
| --- | --- | --- |
| Detection | mAP@0.5, RMSE | Attack Success Rate (ASR), mAP degradation |
| Tracking | MOTA, MOTP, IDF1 | Track loss rate, false track rate, position RMSE under attack |
| Fusion | Sensor utilization rate | Cross-sensor disagreement, existence probability trajectory |
| System | End-to-end latency | Time-to-compromise, recovery time |

## 7. Defensive Countermeasures (Research Output)

| Defense | Application | Method |
| --- | --- | --- |
| Adversarial Training | YOLO v4 | Augment training with EOT-generated maritime patches |
| Feature Squeezing | Camera | Reduce bit depth / spatial smoothing before detection |
| Point Cloud Filtering | Radar/Lidar | Statistical outlier removal before clustering |
| Multi-Sensor Agreement | JIPDA | Require ≥2 sensors for track confirmation; flag single-sensor tracks as tentative |
| Randomized Smoothing | All | Certifiable robustness bounds for clustering + detection |
| Anomaly Detection | Navigation | Detect GPS spoofing via IMU consistency checks |

## 8. Ethical & Safety Considerations

This research plan is designed for defensive purposes:

- Use the open dataset in controlled simulation environments

- Publish vulnerabilities to encourage robust system design

- Coordinate with NTNU Autoferry team for responsible disclosure

- Focus on improving sensor fusion resilience for autonomous passenger ferries